1. Serial communication: HEX format

Port: ttyS5 Baud Rate: 9600

Command return: When the command is correct, the sent character will be returned automatically.

- Set the brightness of the fill light: ( Command format: hexadecimal HEX ) //Only applicable to devices with supplemental lighting

|  | start symbol Fixed value | Brightness value | End symbol Fixed value |
| --- | --- | --- | --- |
| Value/Range | 0x4C | 0x00 – 0x64 | 0x23 |
| The brightness value ranges from 0 to 100; the lower the value, the darker the image. Example: 100% brightness: 4C 64 23 60% brightness: 4C 3C 23 Turn off fill light: 4C 00 23 The query setting is 4C FF 23, which will return the brightness value. |  |  |  |

- Set the status light color (command format: hexadecimal HEX)

|  | red | green | blue |
| --- | --- | --- | --- |
| byte position | 1 | 2 | 3 |
| Value (range) | 0x00-0xFF | 0x00-0xFF | 0x00-0xFF |
| Command example: Red: FF 00 00 Green: 00 FF 00 Blue : 00 00 FF Low-brightness blue: 00 00 64 Lights off: 00 00 00 White: FF FF FF full color：435341573dff It supports 16.77 million colors (full color) and is defined in standard RGB-16 base (similar to the colors used in Photoshop, Android Studio, etc.). |  |  |  |

- Set the status light to flash (command format: hexadecimal HEX)

| Function | byte position | scope | describe |
| --- | --- | --- | --- |
| red | 1 | 0x00 – 0xFF | LED color |
| green | 2 |  |  |
| blue | 3 |  |  |
| Lighting duration | 4 |  | - The unit is 10ms. If the light-on duration is set to 600ms, it should be calculated as: 600ms/10 = 60 (0x3C). - The switching on and off of the lights creates a flashing effect. - The duration of the light on and off can be set according to the speed of the flashing. The typical setting for light-on time is 600ms (0x3C), and light-off time is 400ms (0x28). |
| Lights off duration | 5 |  |  |
| Number of flashes | 6 |  | Set the number of flashes to 0x00. |
| End state | 7 | 0x00 – 0x01 | After the flashing count is complete, the light returns to either a constant on state (0x01) or an off state (0x00). |
| Example : The red light is set to flash every 600ms to turn on and every 400ms to turn off, and then turns off after flashing 100 times. Command: FF 00 00 3C 28 64 00 |  |  |  |